Design of Light Switch

Lab no# 07

Spring 2022

CSE-308L Digital Systems Design lab

Submitted by: Ashfaq Ahmad

Registration No: 19PWCSE1795

Class Section: B

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Student Signature: ______________

Submitted to:

Dr: Ma’am Madeha sheer

June 11, 2022

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

Objective:

- Build a light switch controller such that when the push button is pressed the light if “off” turns “on” and if “on” turns “off”.

Block Diagram:

- Connect the button input to a dip or push button on the S6 board and light output to LED.

Synchronizer Circuit:

These two registers are two D_FFs.

Level to Pulse FSM:

Given register is D_FFs.

Task 01: Design a Level to Pulse Converter:

Description:

For level to pulse first we convert user input (button) to synchronized input by using two D_FFs (we can also use one but two for accurate synchronization).

We can also synchronize user input by simple assigning it to other variable on clock edge in separate module.

Block Diagram:

State Transition Diagram of Level to Pulse converter:

In case of Moore machine,

Source Code:

module L2P_Converter(out,in,clk_100Mhz,reset);

input in,clk_100Mhz,reset;

output reg out;

wire clk_1Mhz,syn_out;

parameter s0=0,s1=1;  //posiible stats . signal level either 1 or 0.

reg state,next_state;

clk_divider cd(clk_1Mhz,clk_100Mhz,reset);

synchronizer syn(syn_out,in,clk_1Mhz,reset);

always @(*)           //star '*' means this block is sensitive to all inputs used in it.

begin

case(state)    //stats diagram code

s0:

if(syn_out)

begin

out=1;

next_state=s1;

end

else

begin

out=0;

next_state=s0;

end

s1:

if(syn_out)

begin

out=0;

next_state=s1;

end

else

begin

out=0;

next_state=s0;

end

endcase

end

//we can also remove given always block and add default case in case statement.

always @(posedge clk_1Mhz)  //reset block

begin

if(reset)

state=s0;

else

state=next_state;

end

endmodule

module clk_divider(clk_1Mhz,clk_100Mhz,reset);  //clock divider

input clk_100Mhz,reset;

output reg clk_1Mhz;

integer c;    //we can also delacre c as a reg.

always @(posedge clk_100Mhz)

if(reset)

begin

c=0;

clk_1Mhz=1;

end

else

begin

c=c+1'b1;

if(c==50000000)

begin

clk_1Mhz=~clk_1Mhz;

c=0;

end

end

endmodule

module D_FF(Q,D,clk_1Mhz,reset);  //D_FF

input D,clk_1Mhz,reset;

output reg Q;

always @(posedge clk_1Mhz)

if(reset)

Q=0;

else

Q=D;

endmodule

module synchronizer(syn_out,in,clk_1Mhz,reset);   //synchronizer

input in,clk_1Mhz,reset;

output syn_out;

wire FF1_out;

D_FF ff1(FF1_out,in,clk_1Mhz,reset);

D_FF ff2(syn_out,FF1_out,clk_1Mhz,reset);

endmodule

Output:

- Input is Dip switch no 1 and reset is dip switch no 8.

- States are s0=0 and s1=1.

In=0;   state=s0; out=0; next state=s0.

In=1;   state=s0; out=1; next state=s1.

In=1;   state=s1; out=0; next state=s1.

In=0;   state=s1; out=0; next state=s0.

Reset state:

Even in=1 but out=0 and state=s0.

Task 02: Design a light switch.

Block Diagram:

States Transition Diagram:

Mealy Machine:

Moore Machine:

Source Code:

module light_switch(switch_out,in,clk_100Mhz,reset);  //light switch main module.

input in,clk_100Mhz,reset;

output reg switch_out;

wire clk_1Mhz,syn_out,L2P_out;

reg state,next_state;

parameter s0=0,s1=1;

clk_divider cd(clk_1Mhz,clk_100Mhz,reset);

synchronizer syn(syn_out,in,clk_1Mhz,reset); //i can also instantiate synchronizer in L2p_converter block.

L2P_Converter l2p(L2P_out,syn_out,clk_1Mhz,reset);

always@(*)     //light switch state digram code

case(state)

s0:

if(L2P_out)

begin

switch_out=1;

next_state=s1;

end

else

begin

switch_out=0;

next_state=s0;

end

s1:

if(L2P_out)

begin

switch_out=0;

next_state=s0;

end

else

begin

switch_out=1;

next_state=s1;

end

endcase

//we can also remove given always block and add default case in case statement.

always@(posedge clk_1Mhz)

if(reset)

state=s0;

else

state=next_state;

endmodule

module clk_divider(clk_1Mhz,clk_100Mhz,reset);  //clock divider

input clk_100Mhz,reset;

output reg clk_1Mhz;

integer c;    //we can also delacre c as a reg.

always @(posedge clk_100Mhz)

if(reset)

begin

c=0;

clk_1Mhz=1;

end

else

begin

c=c+1'b1;

if(c==20000000) //here i reduced value of c so the output clock is not 1_Mhz.

begin

clk_1Mhz=~clk_1Mhz;

c=0;

end

end

endmodule

//I have designed synchronizer without using Flip flops. but funcationality same.

module synchronizer(syn_out,in,clk_1Mhz,reset);   //synchronizer

input in,clk_1Mhz,reset;

output reg syn_out;

reg FF1_out;

always @(posedge clk_1Mhz)

if(reset)

syn_out=0;

else

begin

FF1_out=in;  //here also two FF will be generated. but coding method is different.

syn_out=FF1_out;

end

endmodule

module L2P_Converter(L2P_out,syn_out,clk_1Mhz,reset); //level to pulse converter.

input syn_out,clk_1Mhz,reset;

output reg L2P_out;

parameter s0=0,s1=1;  //posiible stats . signal level either 1 or 0.

reg state,next_state;

always @(*)           //star '*' means this block is sensitive to all inputs used inside it.

begin

case(state)    //stats diagram code of L2P.

s0:

if(syn_out)

begin

L2P_out=1;

next_state=s1;

end

else

begin

L2P_out=0;

next_state=s0;

end

s1:

if(syn_out)

begin

L2P_out=0;

next_state=s1;

end

else

begin

L2P_out=0;

next_state=s0;

end

endcase

end

always @(posedge clk_1Mhz)  //reset block

begin

if(reset)

state=s0;

else

state=next_state;

end

endmodule

Output:

- The input Pin is Dip switch no 1 and reset switch is dip switch no 8.

- States are s0=0 and s1=1.

In=0;   state=s0; out=0; next state=s0.

In=1;   state=s0; out=1; next state=s1.

In=0;   state=s1; out=1; next state=s1.

In=1;   state=s1; out=0; next state=s0.

Reset state:

Even input is one but output is zero and state is s0.