Digital Lock (sequence Detector)

Lab no# 08

Spring 2022

CSE-308L Digital Systems Design lab

Submitted by: Ashfaq Ahmad

Registration No: 19PWCSE1795

Class Section: B

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Student Signature: ______________

Submitted to:

Dr: Ma’am Madeha sheer

June 16, 2022

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

Objective:

- Build an electronic combination lock with reset button, two number buttons (0 and 1), and an unlock output.

Block Diagram: A combinational digital lock has three input buttons for Reset, entering a “0” and entering a “1” and output button UNLOCK and state which shows in which state the machine is currently in. The state output is connected to the seven segment display on the S6BOARD. The Module button in the below diagram is an abstraction of the synchronizer and level to pulse converter from the previous lab. The state transition diagram is given in the following figures.

Note: As there are only two distinct digits in code I,e 0 and 1 so we use two buttons 0 and 1. Here we ignore the off state of both buttons. We can’t use one button to get a sequence like 0110 or 0010 etc by using both on and off state of button. By using one button we can get a sequence like 10101010… Or     01010101 ...  [0 mean off button 1 mean on button].

Task01: Design a sequence detector or digital lock that detect “01011”.

State Transition Diagram:

Moore Machine (non-overlapping):

Output=Unlock

Inputs=0 and 1

Mealy Machine (non-overlapping):

Note: 6 states are also possible.

Source Code:  (Two always block used)

Moore Machine:

module digital_Lock_Moore(out,enable,seg7,zero,one,clk_100Mhz,reset);  //sequence detector 01011

input zero,one,clk_100Mhz,reset;

output [2:0]enable;

output reg out;

output [6:0]seg7;

wire L2P_out0,L2P_out1,clk_5hz;

reg [2:0]state,next_state;

parameter s0=3'd0,s1=3'd1,s2=3'd2,s3=3'd3,s4=3'd4,s5=3'd5;

clk_divider cd(clk_5hz,clk_100Mhz,reset);

L2P_Converter l2p0(L2P_out0,zero,clk_5hz,reset);

L2P_Converter l2p1(L2P_out1,one,clk_5hz,reset);

seven_seg ss(seg7,state);

assign enable=3'b110;

always@(posedge clk_5hz,posedge reset)

begin

if(reset)   //if reset inital state= s0.

state=s0;

else

state=next_state;

end

always @(*)      //Moore Machine

begin

case(state)

s0:

begin

out=0;

if(L2P_out0)

next_state=s1;

else if(L2P_out1)

next_state=s0;

else

next_state=state;

end

s1:

begin

out=0;

if(L2P_out0)

next_state=s1;

else if(L2P_out1)

next_state=s2;

else

next_state=state;

end

s2:

begin

out=0;

if(L2P_out0)

next_state=s3;

else if(L2P_out1)

next_state=s0;

else

next_state=state;

end

s3:

begin

out=0;

if(L2P_out0)

next_state=s1;

else if(L2P_out1)

next_state=s4;

else

next_state=state;

end

s4:

begin

out=0;

if(L2P_out0)

next_state=s3;

else if(L2P_out1)

next_state=s5;

else

next_state=state;

end

s5:

begin

out=1;

if(L2P_out0)

next_state=s1;

else if(L2P_out1)

next_state=s0;

else

next_state=state;

end

default:  //if initally no reset default case will execute. if there is no default case then random case will be executing.

begin

next_state=s0;

out=0;

end

endcase

end

endmodule

module clk_divider(clk_5hz,clk_100Mhz,reset);  //clock divider

input clk_100Mhz,reset;

output reg clk_5hz;

integer c;    //we can also delacre c as a reg.

always @(posedge clk_100Mhz)

if(reset)

begin

c=0;

clk_5hz=1;

end

else

begin

c=c+1'b1;

if(c==10000000)

begin

clk_5hz=~clk_5hz;

c=0;

end

end

endmodule

module D_FF(Q,D,clk_5hz,reset);  //D_FF.

input D,clk_5hz,reset;

output reg Q;

always @(posedge clk_5hz)

if(reset)

Q=0;

else

Q=D;

endmodule

module synchronizer(syn_out,in,clk_5hz,reset);   //synchronizer

input in,clk_5hz,reset;

output syn_out;

wire FF1_out;

//we can also design synchronizer without using D_FFs.

D_FF ff1(FF1_out,in,clk_5hz,reset);

D_FF ff2(syn_out,FF1_out,clk_5hz,reset);

endmodule

module L2P_Converter(L2P_out,in,clk_5hz,reset); //level to pulse converter.

input in,clk_5hz,reset;

output reg L2P_out;

wire syn_out;

synchronizer ss1(syn_out,in,clk_5hz,reset);

parameter s0=0,s1=1;  //posiible stats . signal level either 1 or 0.

reg state,next_state;

always @(*)           //star '*' means this block is sensitive to all inputs used inside it.

begin

case(state)    //stats diagram code of L2P.

s0:

if(syn_out)

begin

L2P_out=1;

next_state=s1;

end

else

begin

L2P_out=0;

next_state=s0;

end

s1:

if(syn_out)

begin

L2P_out=0;

next_state=s1;

end

else

begin

L2P_out=0;

next_state=s0;

end

endcase

end

always @(posedge clk_5hz)  //reset block

begin

if(reset)

state=s0;

else

state=next_state;

end

endmodule

module seven_seg(seg7,state);   //7_segment display

input [2:0]state;

output [6:0] seg7;     // as we have 0 to 4 states only. so only these five states will be used.

assign seg7=(state==3'b000)? 7'b1000000:

(state==3'b001)? 7'b1111001:

(state==3'b010)? 7'b0100100:

(state==3'b011)? 7'b0110000:

(state==3'b100)? 7'b0011001:

(state==3'b101)? 7'b0010010:

(state==3'b110)? 7'b0000010:

(state==3'b111)? 7'b1111000:7'b1111111;

endmodule

Mealy Code:

always@(posedge clk_5hz,posedge reset)

begin

if(reset)  //if reset inital state= s0.

state=s0;

else

state=next_state;

end

always @(*)               //Mealy Machine...

begin

case(state)

s0:

if(L2P_out0)

begin

out=0;

next_state=s1;

end

else

if(L2P_out1)

begin

out=0;

next_state=s0;

end

else

begin

out=out;

next_state=state;

end

s1:

if(L2P_out0)

begin

out=0;

next_state=s1;

end

else

if(L2P_out1)

begin

out=0;

next_state=s2;

end

else

begin

out=out;

next_state=state;

end

s2:

if(L2P_out0)

begin

out=0;

next_state=s3;

end

else

if(L2P_out1)

begin

out=0;

next_state=s0;

end

else

begin

out=out;

next_state=state;

end

s3:

if(L2P_out0)

begin

out=0;

next_state=s1;

end

else

if(L2P_out1)

begin

out=0;

next_state=s4;

end

else

begin

out=out;

next_state=state;

end

s4:

if(L2P_out0)

begin

out=0;

next_state=s1;

end

else

if(L2P_out1)

begin

out=1;

next_state=s0;

end

else

begin

out=out;

next_state=state;

end

default:    //if initally no reset default case will execute. if there is no default case then random case will be executing.

begin

next_state=s0;

out=0;

end

endcase

end

endmodule

The Rest part of code is same as Moore.

UCF File:

Output: (Moore output)

- The input Pins are Dip switch no 1 for 0 input and Dip switch no 2 for 1 input and reset switch is dip switch no 8.

- States are s0=0, s1=1, s2=2, s3=3, s4=4, s5=5.

Initial state=s0; out=0; input= no input.

In=0;   state=s1; out=0;

(Initially it was in s0 when first correct input 1 entered it goes to s1 state).

In=1;   state=s2; out=0;

(First it was in s1 when next correct input 1 entered it goes to s2 state).

In=0;   state=s3; out=0;

(First it was in s2 when next correct input 0 entered it goes to s3 state).

In=1;   state=s4; out=0;

(First it was in s3 when next correct input 1 entered it goes to s4 state).

In=1;   state=s5; out=1;

(First it was in s4 when next correct input 0 entered it goes to s5 state).

Reset state:

Even input is 0 but output is zero and state is s0.

Task 02: Change the functionality of the lock such that it unlocks on the sequence of 11011.

State Transition Diagram:

Moore Machine:

Mealy Machine:

Source Code:   (one always block used)

Moore Machine:

//****************************sequence detector 11011*************************

module digital_lock_Moore(seg7,out,zero,one,clk_100Mhz,reset,enable);

input zero,one,clk_100Mhz,reset;

output [2:0]enable;

output reg out;

output [7:0]seg7;

wire syn_out1,syn_out2,L2P_out0,L2P_out1,clk_1hz;

reg [2:0]state;

parameter s0=3'd0,s1=3'd1,s2=3'd2,s3=3'd3,s4=3'd4,s5=3'd5;

//if i use s0=0 and s1=1 etc. then it give warning becuase these decimal are by default 32 bits and

// state register is 3 bits the warning will be "32-bits truncated to 3  bits".

clk_divider cd(clk_1hz,clk_100Mhz,reset);

synchronizer syn1(syn_out1,zero,clk_1hz,reset);

synchronizer syn2(syn_out2,one,clk_1hz,reset);

L2P_Converter l2p1(L2P_out0,syn_out1,clk_1hz,reset);

L2P_Converter l2p2(L2P_out1,syn_out2,clk_1hz,reset);

seven_seg ss(seg7,state);

assign enable=3'b110;

always@(posedge clk_1hz,posedge reset)   //Moore Machine

begin

if(reset)

begin

state=s0;

out=0;

end

else

case(state)

s0:

begin

out=0;

if(L2P_out0)

state=s0;

else if(L2P_out1)

state=s1;

else

state=state;

end

s1:

begin

out=0;

if(L2P_out0)

state=s0;

else if(L2P_out1)

state=s2;

else

state=state;

end

s2:

begin

out=0;

if(L2P_out0)

state=s3;

else if(L2P_out1)

state=s1;

else

state=state;

end

s3:

begin

out=0;

if(L2P_out0)

state=s0;

else if(L2P_out1)

state=s4;

else

state=state;

end

s4:

begin

out=0;

if(L2P_out0)

state=s0;

else if(L2P_out1)

state=s5;

else

state=state;

end

s5:

begin

out=1;

if(L2P_out0)

state=s0;

else if(L2P_out1)

state=s1;

else

state=state;

end

default:   //if there is no reset then default state will execute . if there is no default and no reset then any random state will execute.

begin

state=s0;

out=0;

end

endcase

end

endmodule

module clk_divider(clk_1hz,clk_100Mhz,reset);  //clock divider

input clk_100Mhz,reset;

output reg clk_1hz;

integer c;    //we can also delacre c as a reg.

always @(posedge clk_100Mhz)

if(reset)

begin

c=0;

clk_1hz=1;

end

else

begin

c=c+1'b1;

if(c==50000000)

begin

clk_1hz=~clk_1hz;

c=0;

end

end

endmodule

module D_FF(Q,D,clk_1hz,reset);  //D_FF.

input D,clk_1hz,reset;

output reg Q;

always @(posedge clk_1hz)

if(reset)

Q=0;

else

Q=D;

endmodule

module synchronizer(syn_out,in,clk_1hz,reset);   //synchronizer

input in,clk_1hz,reset;

output syn_out;

wire FF1_out;

//we can also design synchronizer without using D_FFs.

D_FF ff1(FF1_out,in,clk_1hz,reset);

D_FF ff2(syn_out,FF1_out,clk_1hz,reset);

endmodule

module L2P_Converter(L2P_out,syn_out,clk_1hz,reset); //level to pulse converter.

input syn_out,clk_1hz,reset;

output reg L2P_out;

parameter s0=0,s1=1;  //posiible stats . signal level either 1 or 0.

reg state,next_state; //by default reg size is 32 bits also by default integer size taken as 32 bit so s0 and s1 can store in state and next_state without giving any warning.

always @(*)           //star '*' means this block is sensitive to all inputs used inside it.

begin

case(state)    //stats diagram code of L2P.

s0:            //this is mealy machine.

if(syn_out)

begin

L2P_out=1;

next_state=s1;

end

else

begin

L2P_out=0;

next_state=s0;

end

s1:

if(syn_out)

begin

L2P_out=0;

next_state=s1;

end

else

begin

L2P_out=0;

next_state=s0;

end

endcase

end

//we can also use single always block then no need of next_register. as i used one alwyas block in main module.

always @(posedge clk_1hz)  //reset block

begin

if(reset)

state=s0;

else

state=next_state;

end

endmodule

module seven_seg(seg7,state);   //7_segment display

input [2:0]state;

output [7:0] seg7;     // as we have 0 to 5 states only. so only these six states will be used.

assign seg7=(state==3'b000)? 8'b01000000:  //the last 8th bit for dit.

(state==3'b001)? 8'b01111001:

(state==3'b010)? 8'b00100100:

(state==3'b011)? 8'b00110000:

(state==3'b100)? 8'b00011001:

(state==3'b101)? 8'b00010010:

(state==3'b110)? 8'b00000010:

(state==3'b111)? 8'b01111000:8'b01111111;

endmodule

Mealy Machine:

always@(posedge clk_1hz,posedge reset)   //Mealy Machine

begin

if(reset)    //initial state...

begin

state=s0;

out=0;

end

else

case(state)

s0:

if(L2P_out0)

begin

out=0;

state=s0;  //both input not possible at  a time also we don't need.even if we entered both input at a time it will considered L2p_out1 input.

end

else if(L2P_out1)

begin

out=0;

state=s1;

end

s1:

if(L2P_out0)

begin

out=0;

state=s0;

end

else if(L2P_out1)

begin

out=0;

state=s2;

end

s2:

if(L2P_out0)

begin

out=0;

state=s3;

end

else if(L2P_out1)

begin

out=0;

state=s1;

end

s3:

if(L2P_out0)

begin

out=0;

state=s0;

end

else if(L2P_out1)

begin

out=0;

state=s4;

end

s4:

if(L2P_out0)

begin

out=0;

state=s0;

end

else if(L2P_out1)

begin

out=1;

state=s0;

end

default:    //if initially not reset then default state execute. if there is no reset and no default state then random state execute.

begin

state=s0;

out=0;

end

endcase

end

endmodule

The rest part of code is same as Moore.

UCF file:

Output: (Moore output)

- The input Pins are Dip switch no 1 for 0 input and Dip switch no 2 for 1 input and reset switch is dip switch no 8.

- States are s0=0, s1=1, s2=2, s3=3, s4=4, s5=5.

Initial state=s0; out=0; input= no input.

In=1;   state=s1; out=0;

(Initially it was in s0 when first correct input 1 entered it goes to s1 state).

In=1;   state=s2; out=0;

(First it was in s1 when next correct input 1 entered it goes to s2 state).

In=0;   state=s3; out=0;

(First it was in s2 when next correct input 0 entered it goes to s3 state).

In=1;   state=s4; out=0;

(First it was in s3 when next correct input 1 entered it goes to s4 state).

In=1;   state=s5; out=1;

(First it was in s4 when next correct input 0 entered it goes to s5 state).

Reset state:

Even input is one but output is zero and state is s0.